Table of Contents

Modules Design Code:	2

RAM Design Code:	2

SPI_SLAVE_DESIGN_CODE:	4

TOP_MODULE_DESIGN_CODE:	8

Test Bench:	9

Do File:	12

Constrain File:	12

Encoding:	13

ONE_HOT:	13

Elaborated Design:	13

Synthesis:	13

Implementation:	15

Bitstream File:	16

Gray_Encoding:	16

Elaborated Design:	16

Synthesis:	16

Implementation:	18

Bitstream File:	19

Sequential_Encoding:	20

Elaborated Design:	20

Synthesis:	20

Implementation:	22

Bitstream File:	24

Debug Core:	24

# Modules Design Code:

## RAM Design Code:

module RAM #(

parameter MEM_DEPTH = 256,  // Defines the depth of memory (number of addresses available)

parameter ADDR_SIZE = 8     // Address size (number of bits to address memory locations)

) (

input [9:0] din,            // 10-bit data input (2 bits for control, 8 bits for address/data)

input rx_valid, clk, rst_n, // Control signals: receive valid, clock, and active-low reset

output reg [7:0] dout,      // 8-bit data output (memory read result)

output reg tx_valid         // Transmit valid signal (indicates data is ready to be transmitted)

);

reg [7:0] memory [MEM_DEPTH-1:0]; // Memory array declaration

reg [ADDR_SIZE-1:0] WR_address_bus; // Write address register

reg [ADDR_SIZE-1:0] RD_address_bus; // Read address register

reg flag_read;  // Flag to indicate a pending read operation

reg flag_write; // Flag to indicate a pending write operation

always @(posedge clk) begin

if (~rst_n) begin // Active-low reset handling

dout <= 0;

tx_valid <= 0;

flag_write <= 0;

flag_read <= 0;

RD_address_bus <= 0;

WR_address_bus <= 0;

end

else begin

tx_valid <= 0; // Ensure tx_valid is cleared at the start of each clock cycle

case ({din[9], din[8]}) // Extracting the control bits from din

2'b00: begin

/*

* Case 00: Handle write address request from SPI slave

* If no previous write is pending, store the write address

*/

if (rx_valid && ~flag_write) begin

WR_address_bus <= din[7:0]; // Store write address

flag_write <= 1; // Indicate address is ready to be written on

end

// TODO: Implement SPI Slave communication to set SS_n = 1 after accepting the address

end

2'b01: begin

/*

* Case 01: Handle writing data to memory

* Only write if a previous write address has been set

*/

if (rx_valid && flag_write) begin

flag_write <= 0;

memory[WR_address_bus] <= din[7:0]; // Write data to the address stored

end

end

2'b10: begin

/*

* Case 10: Handle read address request from SPI slave

* If no previous read is pending, store the read address

*/

if (rx_valid && ~flag_read) begin

flag_read <= 1;

RD_address_bus <= din[7:0]; // Store read address

end

end

2'b11: begin

/*

*   Case 11: Handle memory read operation

*   Read data from stored read address and assert tx_valid

*/

if (rx_valid && flag_read) begin

dout <= memory[RD_address_bus]; // Read data from memory

flag_read <= 0;

tx_valid <= 1; // Indicate data is ready to be sent

end

end

endcase

end

end

endmodule

## SPI_SLAVE_DESIGN_CODE:

module SPI_Slave #(

//States

parameter IDLE      ='b000 ,

parameter CHK_CMD   ='b001 ,

parameter WRITE     ='b010 ,

parameter READ_ADD  ='b011 ,

parameter READ_DATA ='b100

)(

//Control Inputs

input SS_n,CLK,rst_n,

//TX Inputs

input tx_valid,

input [7:0] tx_data,

//Protocol Lines

input MOSI,

output reg MISO,

//RX Outputs

output reg rx_valid,

output reg [9:0] rx_data

);

////////////////////////////// Variables ////////////////////////////////////////////////////////////////

// State Variables

reg [2:0] cs,ns;

//Global Variables

reg is_address_received;

reg data_received;

//Tasks Variables:

//  1)SER_TO_PAR

reg [3:0] cycle_counter_SER_TO_PAR;

//  2)PAR_TO_SER

reg [3:0] cycle_counter_PAR_TO_SER;

reg [7:0]Parallel_data_out;

///////////////////////////////////// FSM Algorithm /////////////////////////////////////////////////////

//State Memory

always @(posedge CLK) begin

if(~rst_n)begin

cs <= IDLE ;

end

else begin

cs <= ns   ;

end

end

//Next State Logic

always @(*) begin

case (cs)

IDLE: begin

if (SS_n) begin

ns = IDLE ;

end

else begin

ns = CHK_CMD ;

end

end

CHK_CMD: begin

if (SS_n) begin

ns = IDLE ;

end

else if (MOSI == 0) begin

ns = WRITE ;

end

else if (~is_address_received) begin

ns = READ_ADD ;

end

else begin

ns = READ_DATA ;

end

end

WRITE: begin

if (SS_n) begin

ns = IDLE ;

end else begin

ns = WRITE ;

end

end

READ_ADD: begin

if (SS_n) begin

ns = IDLE ;

end else begin

ns = READ_ADD ;

end

end

READ_DATA: begin

if (SS_n) begin

ns = IDLE ;

end else begin

ns = READ_DATA ;

end

end

default: ns = IDLE ;

endcase

end

//Output Logic

always @(posedge CLK) begin

if(~rst_n)begin

MISO        <= 0 ;

rx_valid    <= 0 ;

rx_data     <= 0 ;

//some internal signals

is_address_received         <= 0 ;

Parallel_data_out           <= 0 ;

cycle_counter_PAR_TO_SER    <= 0 ;

cycle_counter_SER_TO_PAR    <= 0 ;

end

else begin

if (SS_n) begin

communication_ended();

end else if (cs == CHK_CMD) begin

SER_TO_PAR();

end else if (cs == WRITE) begin

SER_TO_PAR();

end else if (cs == READ_ADD) begin

SER_TO_PAR();

end else if (cs == READ_DATA) begin

READ_DATA_TASK();

end

end

end

///////////////////////////////////////////// Tasks /////////////////////////////////////////////////////////

task SER_TO_PAR();

begin

if (cycle_counter_SER_TO_PAR < 10) begin

rx_data <= {rx_data [8:0] , MOSI} ;

cycle_counter_SER_TO_PAR <= cycle_counter_SER_TO_PAR + 1 ;

end

if (cycle_counter_SER_TO_PAR == 9) begin

rx_valid <= 1;

if (cs == READ_ADD) is_address_received <= 1;

else is_address_received <= 0;

end

end

endtask

task PAR_TO_SER();

begin

if (cycle_counter_PAR_TO_SER < 8) begin

MISO <= Parallel_data_out[7];

Parallel_data_out <= Parallel_data_out<<1;

cycle_counter_PAR_TO_SER <= cycle_counter_PAR_TO_SER + 1 ;

end

if(cycle_counter_PAR_TO_SER == 7) begin

data_received <= 0 ;

is_address_received <= 0 ;

end

end

endtask

task READ_DATA_TASK();

begin

if(data_received)begin

PAR_TO_SER();

end

else if (tx_valid) begin

rx_valid <= 0;

Parallel_data_out <= tx_data;

data_received <= 1 ;

end

else begin

SER_TO_PAR();

end

end

endtask

// Handling any time SS_n = 1 (at the end of communication or accidentaly)

task communication_ended();begin

MISO                        <= 0 ;

cycle_counter_PAR_TO_SER    <= 0 ;

cycle_counter_SER_TO_PAR    <= 0 ;

rx_valid                    <= 0 ;

Parallel_data_out           <= 0 ;

end

endtask

endmodule

## TOP_MODULE_DESIGN_CODE:

module SPI#(

parameter MEM_DEPTH = 256,

parameter ADDR_SIZE = 8

) (

input MOSI ,SS_n, CLK , rst_n,

output MISO

);

wire [7:0] tx_data ;

wire [9:0] rx_data ;

wire rx_valid, tx_valid;

SPI_Slave Slave_handler (

.SS_n(SS_n),

.CLK(CLK),

.rst_n(rst_n),

.tx_valid(tx_valid),

.tx_data(tx_data),

.MOSI(MOSI),

.MISO(MISO),

.rx_valid(rx_valid),

.rx_data(rx_data)

);

RAM #(

.MEM_DEPTH(MEM_DEPTH),

.ADDR_SIZE(ADDR_SIZE)

)RAM1 (

.clk(CLK),

.rst_n(rst_n),

.din(rx_data),

.rx_valid(rx_valid),

.dout(tx_data),

.tx_valid(tx_valid)

);

endmodule

# Test Bench:

module SPI_tb();

bit MOSI_tb ,SS_n, CLK , rst_n;

logic MISO_tb;

SPI DUT (

.SS_n(SS_n),

.CLK(CLK),

.rst_n(rst_n),

.MOSI(MOSI_tb),

.MISO(MISO_tb)

);

initial begin

CLK = 0;

forever #1 CLK = ~CLK;

end

logic [7:0] address,data;

initial begin

SS_n = 1;

MOSI_tb = 0;

// Reset the system

reset();

$readmemb("mem.dat",DUT.RAM1.memory);

//////////////////////////////////////////// Normal FulL Scenario //////////////////////////////////////

// Test Case 1: Write Address (WRITE)

SS_n = 0;

@(negedge CLK);

address = 8'b1111_0000;

receive_from_master(10'b00_1111_0000); // Command 00 (WRITE) + 8-bit data

SS_n = 1;

@(negedge CLK);

$stop;

// Test Case 2: Write Data (WRITE)

SS_n = 0;

@(negedge CLK);

address = 8'b1111_0000;

receive_from_master(10'b01_1111_0000); // Command 01 (WRITE) + 8-bit data

SS_n = 1;

@(negedge CLK);

$stop;

// Test Case 3: Read Address (READ_ADD)

SS_n = 0;

@(negedge CLK);

address = 8'b1111_0000;

receive_from_master(10'b10_1111_0000); // Command 10 (READ_ADD) + 8-bit address

SS_n = 1;

@(negedge CLK);

$stop;

// Test Case 4: Read Data (READ_DATA)

SS_n = 0;

@(negedge CLK);

address = 8'b0000_0000;

receive_from_master(10'b11_0000_0000); // Command 11 (READ_DATA) + dummy data

@(negedge CLK);// wait for tx data from RAM

repeat(9) @(negedge CLK); // Wait for 8-bit serialization

SS_n = 1;

@(negedge CLK);

$display("Normal Scenerio Finished ,Now starting Just Reading with self checking");

///////////////////////////////////////// Just Reading with self checking /////////////////////////////////

repeat(10)begin

//Read Address (READ_ADD)

SS_n = 0;

@(negedge CLK);

address = $random;

receive_from_master({2'b10,address}); // Command 10 (READ_ADD) + 8-bit address

SS_n = 1;

@(negedge CLK);

// Test Case 4: Read Data (READ_DATA)

SS_n = 0;

@(negedge CLK);

receive_from_master(10'b11_0000_0000); // Command 11 (READ_DATA) + dummy data

repeat(2)@(negedge CLK);// wait for tx_valid trigger+ another edge spi receive data

if (DUT.RAM1.memory [address] != DUT.Slave_handler.Parallel_data_out) begin

$display("Error in just reading with self checking at time =%d",$time);

$stop;

end

repeat(9) @(negedge CLK); // Wait for 8-bit serialization

SS_n = 1;

@(negedge CLK);

end

$display("Just Reading with self checking Finished with no errors ,Now starting Just Writing with self checking");

//////////////////////////////////// Just Writing with self checking ////////////////////////////////////

repeat(10)begin

//Read Address (Write_ADD)

SS_n = 0;

@(negedge CLK);

address = $random;

data = $random ;

receive_from_master({2'b00,address}); // Command 10 (READ_ADD) + 8-bit address

SS_n = 1;

@(negedge CLK);

// Test Case 4: Read Data (READ_DATA)

SS_n = 0;

@(negedge CLK);

receive_from_master({2'b01,data}); // Command 11 (READ_DATA) + dummy data

@(negedge CLK);// wait for data to be written

if (DUT.RAM1.memory [address] != data ) begin

$display("Error in just Writing with self checking at time =%d",$time);

$stop;

end

SS_n = 1;

@(negedge CLK);

end

$display("Just Writing with self checking Finished with no errors");

$stop;

end

// Reset Task

task reset();

rst_n = 1;

@(negedge CLK);

rst_n = 0;

@(negedge CLK);

rst_n = 1;

endtask

task receive_from_master(logic [9:0] Parallel_data_in_tb);

for (int i = 9; i >= 0; i--) begin

MOSI_tb = Parallel_data_in_tb[i];

@(negedge CLK);

end

endtask

endmodule

# Do File:

# Constrain File:

## Clock signal

set_property -dict { PACKAGE_PIN W5   IOSTANDARD LVCMOS33 } [get_ports CLK]

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports CLK]

## Switches

set_property -dict { PACKAGE_PIN V17   IOSTANDARD LVCMOS33 } [get_ports {rst_n}]

set_property -dict { PACKAGE_PIN V16   IOSTANDARD LVCMOS33 } [get_ports {SS_n}]

set_property -dict { PACKAGE_PIN W16   IOSTANDARD LVCMOS33 } [get_ports {MOSI}]

## LEDs

set_property -dict { PACKAGE_PIN U16   IOSTANDARD LVCMOS33 } [get_ports {MISO}]

## Configuration options, can be used for all designs

set_property CONFIG_VOLTAGE 3.3 [current_design]

set_property CFGBVS VCCO [current_design]

## SPI configuration mode options for QSPI boot, can be used for all designs

set_property BITSTREAM.GENERAL.COMPRESS TRUE [current_design]

set_property BITSTREAM.CONFIG.CONFIGRATE 33 [current_design]

set_property CONFIG_MODE SPIx4 [current_design]

# Encoding:

## ONE_HOT:

### Elaborated Design:

### Synthesis:

#### Utilization Report:

#### Timing Report:

#### Netlist:

// Copyright 1986-2018 Xilinx, Inc. All Rights Reserved.

// --------------------------------------------------------------------------------

// Tool Version: Vivado v.2018.2 (win64) Build 2258646 Thu Jun 14 20:03:12 MDT 2018

// Date        : Thu Mar 13 10:09:09 2025

// Host        : Ziad-Kassem running 64-bit major release  (build 9200)

// Command     : write_verilog D:/Kareem_Wassem/DESIGN/Projects/SPI_Slave_With_Single_Port_RAM/ONE_HOT_NETLIST.v

// Design      : SPI

// Purpose     : This is a Verilog netlist of the current design or from a specific cell of the design. The output is an

//               IEEE 1364-2001 compliant Verilog HDL file that contains netlist information obtained from the input

//               design files.

// Device      : xc7a35ticpg236-1L

// --------------------------------------------------------------------------------

Full file is found including the files and named: ONE_HOT_NETLIST.v

### Implementation:

#### FPGA:

#### Schematic:

#### Timing:

### Bitstream File:

## Gray_Encoding:

### Elaborated Design:

### Synthesis:

#### Utilization:

#### Timing:

#### NETLIST Report:

// Copyright 1986-2018 Xilinx, Inc. All Rights Reserved.

// --------------------------------------------------------------------------------

// Tool Version: Vivado v.2018.2 (win64) Build 2258646 Thu Jun 14 20:03:12 MDT 2018

// Date        : Thu Mar 13 10:45:28 2025

// Host        : Ziad-Kassem running 64-bit major release  (build 9200)

// Command     : write_verilog D:/Kareem_Wassem/DESIGN/Projects/SPI_Slave_With_Single_Port_RAM/GRAY_NETLIST.v

// Design      : SPI

// Purpose     : This is a Verilog netlist of the current design or from a specific cell of the design. The output is an

//               IEEE 1364-2001 compliant Verilog HDL file that contains netlist information obtained from the input

//               design files.

// Device      : xc7a35ticpg236-1L

// --------------------------------------------------------------------------------

Full file is found including the files and named: GRAY_NETLIST.v

### Implementation:

#### FPGA:

#### Schematic:

#### Utilization:

#### Timing:

### Bitstream File:

## Sequential_Encoding:

### Elaborated Design:

### Synthesis:

#### Utilization:

#### Timing:

#### Netlist Report:

// Copyright 1986-2018 Xilinx, Inc. All Rights Reserved.

// --------------------------------------------------------------------------------

// Tool Version: Vivado v.2018.2 (win64) Build 2258646 Thu Jun 14 20:03:12 MDT 2018

// Date        : Thu Mar 13 11:04:24 2025

// Host        : Ziad-Kassem running 64-bit major release  (build 9200)

// Command     : write_verilog D:/Kareem_Wassem/DESIGN/Projects/SPI_Slave_With_Single_Port_RAM/PSEQENTIAL_NETLIST.v

// Design      : SPI

// Purpose     : This is a Verilog netlist of the current design or from a specific cell of the design. The output is an

//               IEEE 1364-2001 compliant Verilog HDL file that contains netlist information obtained from the input

//               design files.

// Device      : xc7a35ticpg236-1L

// --------------------------------------------------------------------------------

Full file is found including the files and named: PSEQUENTIAL_NETLIST.v

### Implementation:

#### FPGA:

#### Schematic:

#### Utilization:

#### Timing:

### Bitstream File:

## Debug Core:

As we found that the synthesis timing for all types of encoding approximately the same , so we choose the gray as the gray won’t need high power in switching case (just change 1 bit ) compared to sequential ,and less area compared to One hot because One hot will add 1 more flipflop.

create_debug_core u_ila_0 ila

set_property ALL_PROBE_SAME_MU true [get_debug_cores u_ila_0]

set_property ALL_PROBE_SAME_MU_CNT 1 [get_debug_cores u_ila_0]

set_property C_ADV_TRIGGER false [get_debug_cores u_ila_0]

set_property C_DATA_DEPTH 1024 [get_debug_cores u_ila_0]

set_property C_EN_STRG_QUAL false [get_debug_cores u_ila_0]

set_property C_INPUT_PIPE_STAGES 0 [get_debug_cores u_ila_0]

set_property C_TRIGIN_EN false [get_debug_cores u_ila_0]

set_property C_TRIGOUT_EN false [get_debug_cores u_ila_0]

set_property port_width 1 [get_debug_ports u_ila_0/clk]

connect_debug_port u_ila_0/clk [get_nets [list CLK_IBUF_BUFG]]

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe0]

set_property port_width 1 [get_debug_ports u_ila_0/probe0]

connect_debug_port u_ila_0/probe0 [get_nets [list CLK_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe1]

set_property port_width 1 [get_debug_ports u_ila_0/probe1]

connect_debug_port u_ila_0/probe1 [get_nets [list MISO_OBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe2]

set_property port_width 1 [get_debug_ports u_ila_0/probe2]

connect_debug_port u_ila_0/probe2 [get_nets [list MOSI_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe3]

set_property port_width 1 [get_debug_ports u_ila_0/probe3]

connect_debug_port u_ila_0/probe3 [get_nets [list rst_n_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe4]

set_property port_width 1 [get_debug_ports u_ila_0/probe4]

connect_debug_port u_ila_0/probe4 [get_nets [list SS_n_IBUF]]

set_property C_CLK_INPUT_FREQ_HZ 300000000 [get_debug_cores dbg_hub]

set_property C_ENABLE_CLK_DIVIDER false [get_debug_cores dbg_hub]

set_property C_USER_SCAN_CHAIN 1 [get_debug_cores dbg_hub]

connect_debug_port dbg_hub/clk [get_nets CLK_IBUF_BUFG]